SleepNest

AI-Powered Baby Room Monitoring System

Technical & UX Evaluation Report

| Module | IT4021 — IoT Systems |
| --- | --- |
| Version | SleepNest v1.0 |
| Date | 2026-05-01 |
| Hardware | Arduino Uno + ESP32 + INMP441 microphone |
| Backend | Node.js 18 + Express + MongoDB Atlas |
| Frontend | React 18 + Vite + Tailwind CSS + Chart.js |
| ML Stack | scikit-learn (RF, IF, K-Means) + ONNX + Azure GPT-5.2 |

# Table of Contents

1.  Problem Definition and Context  ............  3

2.  Dataset Description and Preprocessing  ............  5

3.  UX Design Rationale  ............  8

4.  Visual Analytics Design Decisions  ............  11

5.  Chatbot / Agent Architecture  ............  14

6.  Decision-Support Capabilities  ............  17

7.  Limitations and Future Improvements  ............  19

# 1. Problem Definition and Context

## 1.1  Motivation

Infant monitoring is a critical concern for new parents and caregivers. Traditional baby monitors provide a basic audio/video feed but offer no quantitative environmental analysis, no automated state classification, and no actionable intelligence. Parents must manually interpret audio and visual cues — a cognitively demanding task, especially during night-time hours.

SleepNest addresses this gap by fusing low-cost sensor hardware with machine learning inference and natural language AI to deliver a continuous, quantified picture of the baby's room environment. The system automatically classifies the baby's state (QUIET / LIGHT_ACTIVITY / RESTLESS / CRYING), detects anomalies, identifies behavioural clusters, and allows caregivers to query conditions in plain English.

## 1.2  Problem Statement

The core problem is three-fold:

Situational Awareness: Caregivers cannot continuously observe the infant room without a system that automatically aggregates multi-sensor data into a single comfort metric and state label.

Alert Fatigue: Threshold-based systems generate noisy alerts. A machine-learning approach is needed to distinguish genuine distress from normal noise.

Information Accessibility: Raw sensor streams are meaningless to non-technical users. The system must translate data into human-readable insights and support natural language queries.

## 1.3  Research Context

SleepNest is built for the IT4021 IoT Systems module. It demonstrates the full stack of an IoT pipeline: physical sensing, embedded firmware, cloud connectivity, ML inference, and interactive visualisation. The system is deployed as a real prototype with live hardware.

## 1.4  System Scope

| Component | Scope | Technology |
| --- | --- | --- |
| Sensing Layer | Temperature, humidity, motion, light, audio | Arduino Uno + ESP32 + INMP441 |
| Connectivity | WiFi WebSocket stream to backend | ESP32 802.11n |
| Backend | REST API, WebSocket server, ML inference, DB writes | Node.js 18 + Express + ws |
| Persistence | Batch write every 30 s, TTL 7 days | MongoDB Atlas |
| ML Pipeline | Classification, anomaly detection, clustering, trend | scikit-learn + ONNX |
| Frontend | Real-time dashboard, analysis pages, chatbot | React 18 + Vite + Tailwind |
| AI Assistant | Natural language Q&A about live conditions | Azure OpenAI GPT-5.2 |

## 1.5  System Architecture

Data flows from physical hardware through an ESP32 gateway to a Node.js server and ultimately to the React dashboard. The architecture is illustrated below:

Arduino Uno  (DHT11 · PIR · LDR)
     │  UART Serial @ 9600 baud
     ▼
ESP32  (WiFi gateway + INMP441 I2S microphone)
     │  WebSocket JSON over WiFi
     ▼
Node.js Server  (server.js · port 3007)
     ├─► Format adapter  (adaptESP32DataFormat)
     ├─► SoundProcessor  (energy · stability · trend)
     ├─► MLPredictor     (ONNX Random Forest · ~3 ms)
     ├─► DataBuffer      (30 s batch flush → MongoDB)
     └─► HTTP + WebSocket → React Frontend
              ├─► SensorContext  (polls /api/devices every 2 s)
              ├─► Real-time charts (Chart.js)
              ├─► Analysis pages (Trend · Correlation · Anomaly · Clustering)
              └─► SleepNest AI Chatbot  (Azure GPT-5.2)

## 1.6  Hardware Components

| Component | Role | Interface |
| --- | --- | --- |
| Arduino Uno | DHT11 temperature + humidity, PIR motion, LDR light | UART Serial → ESP32 |
| ESP32 | WiFi gateway, I2S audio capture and pre-processing | WebSocket → Node.js |
| INMP441 | MEMS microphone — captures room audio at 16 kHz | I2S: WS=GPIO15, SCK=GPIO14, SD=GPIO32 |
| DHT11 | Temperature (°C) + Humidity (%) | GPIO4 on Arduino Uno |
| PIR Sensor | Passive infrared motion detection | GPIO5 on Arduino Uno |
| LDR | Ambient light level (0–1023 ADC) | A0 on Arduino Uno |

# 2. Dataset Description and Preprocessing

## 2.1  Dataset Overview

SleepNest uses two datasets: a synthetic training dataset and a real-world MongoDB export used for anomaly detection and clustering analysis.

| Dataset | Rows | Purpose | Source |
| --- | --- | --- | --- |
| sleepnest_dataset.csv | 10,000 | RF classifier training | Synthetic (generate_dataset.py) |
| sleepnest_final_dataset.csv | 10,000 | Cross-validation / analysis | Synthetic with real calibration |
| sleepnest.readings.csv | 3,313 | Anomaly + clustering training | Live MongoDB export (Apr 24–26 2026) |

## 2.2  Synthetic Dataset Generation

The 10,000-row training dataset was generated using generate_dataset.py. Each class (QUIET, LIGHT_ACTIVITY, RESTLESS, CRYING) has 2,500 rows with the following design decisions to ensure realistic, non-trivially-separable data:

Calibrated profiles: base temperature ~32 °C, humidity 55–60%, light 0–560 ADC — matched to real room conditions from the MongoDB export.

Borderline rate = 25%: 25% of each class has sensor values near class boundaries (e.g. QUIET samples with sound.avg near 65–75), simulating label ambiguity.

Outlier rate = 10%: 10% of rows have random spikes in any sensor, reflecting real-world noise.

Label noise = 6%: 6% of rows are deliberately mislabelled to the adjacent class, simulating real-world annotation imperfection.

Gaussian noise applied to all continuous features.

Circadian variation: temperature, humidity, and light vary sinusoidally with time of day to reflect diurnal patterns.

soundHist deliberately noisy: histogram buckets do not perfectly align with sound.avg to avoid overfit on derived features.

## 2.3  Feature Engineering

The following 19 features are computed from the raw sensor window and used by the Random Forest model:

| Feature | Description | Importance |
| --- | --- | --- |
| sound.avg | Mean sound level in 15-sample window (0–1023) | High |
| sound.max | Peak sound level in window | High |
| sound.last | Most recent sound reading | High |
| sound_range | sound.max − sound.avg (spread) | Medium |
| sound_stability | sound.avg / (sound.max + 1) — inverse volatility | Medium |
| soundHist.QUIET | Count of QUIET classifications in window | Medium |
| soundHist.LIGHT_ACTIVITY | Count of LIGHT_ACTIVITY in window | Medium |
| soundHist.RESTLESS | Count of RESTLESS in window | Medium |
| soundHist.CRYING | Count of CRYING in window | High |
| hist_cry_ratio | CRYING count / total samples | High |
| motion.samplesActive | Number of samples with motion = 1 | Low |
| motion.durationMs | Cumulative motion duration in window (ms) | Low |
| motion_active_ratio | samplesActive / sampleCount | Low |
| temp.avg | Mean temperature in window (°C) | Low |
| humidity.avg | Mean humidity in window (%) | Low |
| light.avg | Mean light level in window (ADC) | Low |
| comfort.avg | Mean computed comfort score (0–100) | Medium |
| hour_sin | Cyclical encoding of hour: sin(2π·h/24) | Low |
| hour_cos | Cyclical encoding of hour: cos(2π·h/24) | Low |

## 2.4  Preprocessing Pipeline

The preprocessing pipeline applied before model training:

Timestamp parsing: ISO 8601 strings → datetime objects, then sinusoidal hour encoding.

Derived feature computation: sound_range, sound_stability, hist_cry_ratio, motion_active_ratio.

Label encoding: CRYING=0, LIGHT_ACTIVITY=1, QUIET=2, RESTLESS=3.

Train/test split: 80% training, 20% test, stratified by class.

StandardScaler: applied inside cross-validation folds to prevent data leakage; Random Forest is scale-invariant but scaler is used for SVM and isolation forest.

NaN handling: missing values filled with column median for the real-world dataset.

Label normalisation: inconsistent labels from the real MongoDB export (e.g. 'Quiet', 'Light Activity') mapped to canonical form.

label_map = {
    'Quiet': 'QUIET',  'Light Activity': 'LIGHT_ACTIVITY',
    'Restless': 'RESTLESS',  'CRYING': 'CRYING',
}
df['sound.event'] = df['sound.event'].map(label_map).fillna('QUIET')

## 2.5  Class Distribution

| Class | Training Count | % of Total | Sound Range (ADC) |
| --- | --- | --- | --- |
| QUIET | 2,500 | 25% | 0 – 70 |
| LIGHT_ACTIVITY | 2,500 | 25% | 70 – 250 |
| RESTLESS | 2,500 | 25% | 250 – 400 |
| CRYING | 2,500 | 25% | > 400 |

## 2.6  Comfort Score Formula

The comfort score (0–100) is computed consistently both server-side (db/DataBuffer.js) and client-side (utils/comfort.js) to ensure the model's training features match inference-time inputs:

tempScore  = max(0, 100 − |temp − 20°C| × 8)
humScore   = max(0, 100 − |humidity − 50%| × 2.5)
soundScore = max(0, 100 − (soundLevel / 1023) × 100)

comfort    = tempScore × 0.35 + humScore × 0.25 + soundScore × 0.40
            − motionPenalty    [0 / 5 / 10 / 25 pts based on duration]

Design Note Temperature ideal is 20 °C, not the ~32 °C seen in the real data. This means the real room is consistently penalised on the temp component (~56 pts), which is intentional — it reflects a warm tropical room that is not ideal for an infant. Humidity ideal is 50% with a gentler penalty.

# 3. UX Design Rationale

## 3.1  User Personas

Three distinct user personas were identified, each with different monitoring needs and technical literacy:

| Persona | Context | Primary Need | Key UX Response |
| --- | --- | --- | --- |
| Primary Caregiver (Parent) | Home, monitoring infant during sleep/awake periods | Instant status assessment — is the baby OK? | BabySitter View: full-screen comfort arc, colour-coded state |
| Babysitter / Relative | Unfamiliar with the baby's patterns | Simple clear alert with clear action guidance | Prominent alert banner with explicit text instructions |
| Technical User (Developer) | Debugging, tuning thresholds, ML analysis | Raw data, model performance, trend analysis | History page, Settings, Analysis sub-pages, API endpoints |

## 3.2  Information Architecture

The frontend is organised into two tiers of information density:

Primary tier — BabySitter View (/babysitter): Single-screen, glanceable. Designed for rapid situational assessment. Full-screen SVG comfort arc, state badge, alert banner.

Secondary tier — Dashboard pages (/overview, /monitoring, /sleep, /alerts, /history, /settings): Progressive disclosure. Detailed metrics for users who want deeper insight.

Analysis tier — /analyse/trend, /correlation, /anomaly, /clustering: Data science pages for advanced users.

## 3.3  BabySitter View — Design Decisions

The BabySitter View is the default landing page (/). Key design decisions:

Comfort arc (SVG): A 270° arc gauge translates the 0–100 comfort score into a spatial, pre-attentive visual encoding. The arc fill angle is proportional to comfort %, with a glowing end cap. Colour maps to state (green=OK, amber=restless, red=crying).

Colour-coded theme: The entire page background gradient shifts with state — emerald for resting, amber for restless, red for crying. This enables peripheral monitoring without reading text.

Alert banner: Explicit plain-English instruction for each state (e.g. 'Baby is crying! Please check immediately.'). Colour-coded dot with pulsing animation for CRYING.

Compact sensor strip: Temperature, humidity, sound, light, motion displayed in a bottom strip — visible at a glance without scrolling.

Dark mode support: All themes have dark variants for night monitoring.

## 3.4  Navigation Design

A persistent left sidebar provides navigation. Design choices:

Icon + label sidebar: Reduces cognitive load — icons enable rapid recognition even at reduced sizes.

Active state highlight: Current page indicated by background colour and left border accent.

Collapsible on mobile: Sidebar collapses to icon-only on narrow screens.

Top bar: Shows current device ID, connection status (green/red dot), last update timestamp, and dark mode toggle.

## 3.5  Colour System and Typography

| State | Colour | Usage |
| --- | --- | --- |
| QUIET / OK | #3B9E72 (Emerald) | Comfort arc, status badge, positive indicators |
| LIGHT_ACTIVITY | #5A52E0 (Indigo) | Active but calm state |
| RESTLESS / Warning | #F59E0B (Amber) | Warning state, motion detected |
| CRYING / Critical | #EF4444 (Red) | Critical alert, requires attention |
| Background dark | #0F1117 | Dark mode base — reduces eye strain at night |
| Card dark | #1E2236 | Dark mode panels |
| Accent blue | #1A5676 | Headings, interactive elements |

Typography: Tailwind default font stack (system fonts) at 11–12 pt body, 14–18 pt headings. Code blocks use Courier New. Numerical readouts use tabular numbers (font-variant-numeric: tabular-nums) to prevent layout shifts during live updates.

## 3.6  Responsiveness and Accessibility

Tailwind CSS utility classes provide responsive breakpoints (sm/md/lg/xl).

Colour is never the sole differentiator — text labels accompany all coloured indicators.

The comfort arc includes a central numerical % for users who cannot distinguish colours.

Alert banner text is explicit and actionable, not just an icon.

Dark mode is user-togglable and persists via React state (extendable to localStorage).

# 4. Visual Analytics Design Decisions

## 4.1  Overview — Design Philosophy

SleepNest applies Shneiderman's Visual Information Seeking Mantra: 'Overview first, zoom and filter, then details on demand.' The BabySitter View provides the overview; the dashboard sub-pages provide detail. The Analysis pages provide the deepest analytical layer.

## 4.2  Real-Time Chart Design (Monitoring Page)

The Monitoring page renders four synchronised line charts using Chart.js:

Temperature (°C), Humidity (%), Sound Level (0–1023), and Motion (binary) all share the same time axis, enabling cross-sensor correlation at a glance.

Rolling window: last 60 readings (~2 minutes) are displayed. Older data scrolls off to prevent visual clutter.

Smooth line tension = 0.3 for continuous signals; stepped interpolation for motion (binary state).

Y-axis autoscales per sensor with sensible min/max hints (e.g. temperature 15–40 °C).

Colour coding matches the system palette: green=temperature, blue=humidity, amber=sound, orange=motion.

Chart.js animation disabled (animation: false) for live feeds to prevent lag during rapid 2-second updates.

## 4.3  Trend Analysis Page

The Trend Analysis page implements in-browser statistical analysis:

Input: last 80 readings from SensorContext rolling history.

7-point rolling average: applied first to smooth noise. Window of 7 chosen to balance responsiveness vs smoothing.

Linear regression (Ordinary Least Squares, implemented in JavaScript): fitted to smoothed data. Outputs slope (trend direction) and R² (goodness of fit).

Visual encoding: raw data as light scatter dots, smoothed line in primary colour, regression line as dashed overlay.

Slope interpretation: shown as trend arrow (▲ rising / ▼ falling / → stable) with magnitude label.

Design choice: regression is recalculated client-side every 2 s without a server round-trip, keeping latency minimal.

## 4.4  Correlation Analysis Page

The Correlation page displays Pearson's r between sensor pairs:

Scatter plot: x = sound level, y = comfort score. Points coloured by ML class (QUIET/LIGHT_ACTIVITY/RESTLESS/CRYING) — this reveals the class-specific clustering of the correlation.

Pearson r displayed prominently: Sound vs Comfort shows r = −0.967 (very strong negative correlation), which validates the comfort formula design.

Multiple correlation cards: Temperature vs Comfort, Humidity vs Comfort are also displayed for environmental context.

Design note: correlation is computed from in-session history only (up to 500 readings), giving a session-specific view. Historical correlations from MongoDB are available via /api/db/readings.

## 4.5  Anomaly Detection Page

The Anomaly Detection page visualises the Isolation Forest classifier output:

Timeline chart: x = time, y = anomaly score. Readings classified as anomalies (score < −0.1) shown in red, normal readings in green.

Distribution histogram: anomaly score distribution to show the separation between normal and anomalous readings.

Anomaly log: list of recent anomaly events with timestamp, sensor values, and ML class.

Design decision: Isolation Forest trained only on QUIET readings means RESTLESS and CRYING are anomalies by definition — this correctly flags distress events as 'outside normal sleep behaviour'.

## 4.6  Clustering / Behaviour Patterns Page

The Clustering page presents K-Means behaviour patterns:

Cluster summary cards: one card per cluster showing mean comfort, sound range, and count.

PCA 2D scatter: 6 features projected to 2 dimensions via PCA, coloured by cluster assignment. Generated by analysis_clustering.py and served as static PNG from model/report/.

Elbow + Silhouette chart: shows k=3 as optimal cluster count.

Temporal cluster distribution chart: shows how cluster membership changes over time (e.g. more Cluster 0 during night hours).

Live assignment: current reading is classified against the saved cluster metadata at each poll, showing which cluster the baby is currently in.

## 4.7  Sleep Quality Page

The Sleep page provides a session-level sleep quality summary:

Sleep score (0–100): weighted average of comfort scores across the session, penalised for RESTLESS/CRYING events.

State distribution donut chart: proportion of time in each sound class, giving a visual summary of sleep continuity.

Event timeline: vertical timeline of motion and sound events during the session.

Design rationale: aggregated view avoids overwhelming parents with per-reading data; focuses on session-level patterns.

# 5. Chatbot / Agent Architecture

## 5.1  Overview

SleepNest includes a conversational AI assistant ('SleepNest AI') powered by Azure OpenAI GPT-5.2. The chatbot is embedded as a floating panel (bottom-right corner) accessible from every page in the frontend.

The chatbot answers natural language questions about the baby's current environmental conditions, recent events, and ML analysis results. It is designed to be a decision-support tool — it provides contextualised answers grounded in live sensor data, not generic responses.

## 5.2  Architecture Components

User question
     ↓
ChatBot.jsx  (React floating panel)
     ↓  POST /api/chat  { question, sensorSnapshot, history }
server.js  /api/chat handler
     ├─► Build system prompt  (injected with live sensor values)
     ├─► Append conversation history  (last 6 turns)
     └─► Call Azure OpenAI API
              Resource: dilmivihara715-9367-resource.services.ai.azure.com
              Model:    gpt-5.2-chat  (version 2026-02-10)
              Limit:    500,000 tokens/min · 5,000 req/min
     ↓
ChatBot.jsx  renders streamed/complete response

## 5.3  System Prompt Design

The system prompt is rebuilt on every request to inject the latest sensor snapshot. It defines three aspects of the assistant's behaviour:

Role definition: 'You are SleepNest AI, a caring and knowledgeable baby room monitoring assistant...'

Sensor context injection: all current sensor values are embedded in the system prompt — temperature, humidity, sound level, sound class, comfort score, motion state, light level, ML prediction, anomaly status, cluster assignment.

Behavioural constraints: instructions to keep responses concise (2–4 sentences), avoid speculation beyond available data, escalate to 'please consult a medical professional' for health concerns.

Example system prompt structure:

SYSTEM:
You are SleepNest AI, a caring baby room monitoring assistant.
Current conditions (updated every 2 seconds):
  - Temperature    : 32.3 °C
  - Humidity       : 57.4 %
  - Sound Level    : 84 / 1023 (ADC)
  - Sound Class    : LIGHT_ACTIVITY
  - Comfort Score  : 61 / 100
  - Motion         : inactive
  - Light Level    : 312 ADC
  - ML Prediction  : LIGHT_ACTIVITY (confidence 0.87)
  - Anomaly        : normal
  - Behaviour Cluster: Cluster 1 — Light Activity
Answer the parent's question in 2–4 sentences, grounded in the data above.

## 5.4  Conversation History Management

The frontend maintains a conversation array of { role: 'user'|'assistant', content: string } objects.

Only the last 6 turns (3 user + 3 assistant) are sent to the API to limit token usage while preserving short-term context.

History is stored in React component state — it is session-scoped and resets on page refresh.

The system prompt is prepended fresh on every call with the latest sensor snapshot, so stale context is never an issue.

## 5.5  Azure OpenAI Configuration

| Parameter | Value | Rationale |
| --- | --- | --- |
| Model | gpt-5.2-chat | Latest available GPT model in deployment |
| API Version | 2026-02-10 | Supports max_completion_tokens parameter |
| max_completion_tokens | 500 | Limit response length; concise answers preferred |
| temperature | 0.4 | Lower = more factual, grounded responses |
| History window | 6 turns (3+3) | Balance context vs token cost |
| Rate limit | 500k tokens/min | Sufficient for real-time interactive use |

Note on max_completion_tokens GPT-5.2 requires max_completion_tokens instead of the legacy max_tokens parameter. This is a breaking change from GPT-4 and must be handled in the API request body. Using max_tokens with GPT-5.2 causes a 400 error.

## 5.6  Example Interactions

| User Question | Expected Response Type |
| --- | --- |
| Is the room safe for the baby right now? | Comfort score + threshold assessment |
| What is the current sound level? | Numeric value + dB equivalent + class |
| Was the baby crying recently? | Recent CRYING events from session history |
| Is the temperature too high? | Temperature vs 18–22 °C ideal range |
| What cluster is the baby in? | K-Means cluster label + description |
| Any anomalies detected? | Isolation Forest score + status |
| What should I do if the baby is crying? | Actionable parenting guidance + escalation |

# 6. Decision-Support Capabilities

## 6.1  Overview

SleepNest provides four distinct decision-support capabilities, each addressing a different aspect of the caregiver's information need:

| Capability | Method | Output | Decision Supported |
| --- | --- | --- | --- |
| Sound Classification | Random Forest (ONNX) | QUIET / LIGHT_ACTIVITY / RESTLESS / CRYING | Immediate alert — is action needed? |
| Anomaly Detection | Isolation Forest | Normal / Anomaly + score | Is this reading unusual for this baby? |
| Behaviour Clustering | K-Means (k=3) | Cluster 0/1/2 + description | What is the baby's typical pattern? |
| Trend Analysis | Linear regression + rolling avg | Slope, R², direction arrow | Is the environment improving or worsening? |
| Comfort Score | Weighted formula | 0–100 score + status label | Single aggregated environmental quality metric |
| AI Chatbot | Azure GPT-5.2 | Natural language answer | Any question in plain English |

## 6.2  ML Classification Pipeline

Every sensor reading is passed through a 3-stage classification pipeline:

Stage 1 — Format adaptation: adaptESP32DataFormat() normalises the ESP32 JSON payload to the legacy field names expected by SoundProcessor and MLPredictor. Amplitude is scaled from 0–4095 to 0–1023.

Stage 2 — Rolling window: MLPredictor maintains a 15-sample SensorWindow per device. Features are extracted from the full window (means, maxima, histograms, ratios).

Stage 3 — ONNX inference: the 19-feature vector is fed into the Random Forest ONNX model. Inference latency is ~3–5 ms. The result includes both the predicted class and per-class probabilities.

Fallback: if the ONNX model is unavailable, threshold-based classification is used as a fallback (configurable via config.json).

## 6.3  Alert Decision Logic

Alerts are generated by an edge-triggered state machine:

if currentState !== previousState:
    if state in ['RESTLESS', 'CRYING']:
        if now - lastAlertTime > alertCooldown (5000 ms):
            → write to events collection (MongoDB)
            → append to in-memory alertLog
            → push to frontend timeline

Edge-triggering ensures that sustained CRYING generates one alert, not one per 2-second reading. The 5-second cooldown prevents rapid state oscillations from flooding the alert log.

## 6.4  Comfort Score as Decision Signal

The comfort score aggregates three environmental dimensions into a single 0–100 value with explicit status labels:

| Comfort Range | Status Label | Interpretation |
| --- | --- | --- |
| 85 – 100 | Baby is Deeply Resting | Optimal conditions — no action needed |
| 70 – 84 | Baby is Sleeping Well | Good conditions — monitor passively |
| 55 – 69 | Baby in Light Sleep | Slightly suboptimal — check if waking |
| 40 – 54 | Baby is Restless | Conditions degraded — investigate cause |
| 0 – 39 | Baby Needs Attention | Critical — immediate caregiver action |

## 6.5  Threshold Configurability

Sound thresholds are configurable at runtime via the Settings page or the /api/settings/thresholds endpoint, and are persisted to config.json. This supports environment-specific tuning:

Different rooms have different ambient noise floors.

Different babies have different baseline cry volumes.

The /api/devices/:deviceId/auto-tune endpoint automatically adjusts sensitivity based on recent data distribution.

The /api/devices/:deviceId/calibrate endpoint recalibrates the baseline noise floor for the I2S microphone.

## 6.6  MongoDB as Decision Audit Trail

All state-change events are written to the events collection immediately (not batch-buffered). This creates an audit trail of every CRYING, RESTLESS, or motion event with timestamp, sensor values, and ML classification. Caregivers can query this trail to answer questions like:

'How many times did the baby cry last night?'

'At what time did the baby last wake up?'

'Was the room too warm during the 2 AM feeding?'

# 7. Limitations and Future Improvements

## 7.1  Known Limitations

### 7.1.1  Hardware Constraints

| Limitation | Impact | Severity |
| --- | --- | --- |
| DHT11 sensor accuracy ±2 °C, ±5% RH | Temperature and humidity readings have significant error bands | Medium |
| Single microphone (INMP441) with no directional filtering | Cannot distinguish baby crying from other room sounds (TV, conversations) | High |
| PIR motion sensor does not detect fine motor movements | Breathing and small limb movements are not captured | High |
| Arduino Uno limited to 9600 baud UART | Restricts sensor sampling rate to ~2 s intervals | Low |
| ESP32 WiFi dependency | System fails entirely if WiFi drops | High |
| No camera | Visual state (eyes open/closed, position) not captured | Medium |

### 7.1.2  Software and ML Limitations

| Limitation | Impact | Severity |
| --- | --- | --- |
| Synthetic training data | Model may not generalise to all baby rooms and microphone placements | High |
| Isolation Forest trained on one session's data | Anomaly baseline is time-limited; does not adapt to long-term changes | Medium |
| K-Means requires manual k=3 choice | Optimal k may differ for different babies/rooms | Low |
| In-session history only (500 readings) | Frontend analytics are reset on page refresh | Medium |
| No cry type classification | Cannot distinguish hunger / pain / overtiredness from sound alone | High |
| ONNX model features are static | Adding new sensors requires retraining and redeployment | Medium |
| Azure GPT-5.2 API dependency | Chatbot fails if Azure endpoint is unreachable; no offline fallback | Medium |
| No user authentication | Dashboard is publicly accessible — not suitable for production deployment | High |
| Single-device only | System assumes one ESP32; multi-room monitoring not supported | Medium |

### 7.1.3  UX Limitations

No push notifications: alerts are only visible in the browser tab — parents who navigate away miss alerts.

No persistent dark mode: dark mode state resets on page reload (not persisted to localStorage).

Chatbot history is session-scoped: conversation resets on refresh.

No mobile app: the dashboard is a web app only — it works on mobile browsers but lacks native push notifications.

No user onboarding: first-time users receive no guided tour or explanation of the comfort score formula.

## 7.2  Future Improvements

### 7.2.1  Hardware Enhancements

Replace DHT11 with SHT31 or BME280 for ±0.2 °C / ±2% RH accuracy.

Add a camera module (OV2640 or Raspberry Pi Camera) for visual state detection (sleep position, eye state).

Add a CO₂ sensor (SCD40) to monitor air quality — elevated CO₂ correlates with poor sleep quality.

Replace PIR with mmWave radar (LD2450) for fine breathing and micro-movement detection.

Add a second microphone for spatial audio and direction-of-arrival processing to distinguish baby sounds from background noise.

### 7.2.2  ML Model Improvements

Train on real labelled baby audio data (e.g. Donate-a-Cry corpus or equivalent) rather than synthetic data.

Implement an online learning component: the model retunes on labelled data from the specific baby it monitors.

Extend classification to cry type recognition (hunger / pain / tiredness) using spectral audio features (MFCCs).

Replace Isolation Forest with an online/streaming anomaly detector (e.g. RRCF — Robust Random Cut Forest) that adapts to seasonal baselines.

Add sleep stage classification (NREM/REM) using motion + audio fusion.

### 7.2.3  Backend and Data Engineering

Implement user authentication (JWT + bcrypt) before any production deployment.

Add a WebSocket push channel from server to frontend to replace the 2-second polling loop (reduces latency from 2 s to <100 ms).

Extend MongoDB schema with baby_profile collection (name, birth date, usual sleep schedule) to personalise analysis.

Implement long-term trend analytics: sleep duration, crying frequency per week, comfort score moving averages.

Add data export (CSV, PDF report) for sharing with paediatricians.

Reduce DataBuffer flush interval dynamically (e.g. 10 s when CRYING, 60 s when QUIET) to balance latency vs write cost.

### 7.2.4  UX Enhancements

Progressive Web App (PWA) manifest: enable 'Add to Home Screen' on mobile with native-like full-screen experience.

Push notifications via Web Push API or Firebase Cloud Messaging: alert parents even when the tab is in the background.

Persist dark mode and settings preferences to localStorage or a user profile in MongoDB.

Add a guided onboarding flow for first-time setup (WiFi config, threshold calibration walkthrough).

Internationalisation (i18n): support for multiple languages via react-i18next.

### 7.2.5  AI Chatbot Improvements

Implement a local fallback LLM (e.g. Ollama + Llama 3) for offline operation when Azure is unreachable.

Add function-calling / tool use: allow GPT to query MongoDB history directly to answer time-based questions ('How many CRYING events happened last night?').

Persist conversation history to MongoDB for cross-session continuity.

Add voice input/output for hands-free caregiver interaction.

## 7.3  Summary Assessment

SleepNest successfully demonstrates the full IoT + ML + AI pipeline from physical sensing to intelligent decision support. The prototype proves the concept of multi-sensor fusion for infant monitoring and validates the technical feasibility of running ONNX ML inference in Node.js at production-grade latency (~3–5 ms per prediction).

The primary limitation of the current system is reliance on synthetic training data and single-device scope. The most impactful near-term improvement would be training the classifier on real annotated baby audio, adding push notifications, and implementing authentication before any deployment beyond a home prototype.

Key Metrics Summary Random Forest accuracy: 92.8% ¦ F1 macro: 0.928 Isolation Forest contamination: 8% ¦ Anomaly score threshold: −0.1 K-Means silhouette score: 0.424 (k=3) MongoDB write reduction: 15× (1,800 → 120 writes/hour) ONNX inference latency: ~3–5 ms per prediction Frontend poll interval: 2 seconds ¦ Stale threshold: 60 seconds

# References

[1]  Breiman, L. (2001). Random Forests. Machine Learning, 45(1), 5–32.

[2]  Liu, F. T., Ting, K. M., & Zhou, Z. H. (2008). Isolation Forest. IEEE International Conference on Data Mining (ICDM), 413–422.

[3]  Lloyd, S. P. (1982). Least squares quantization in PCM. IEEE Transactions on Information Theory, 28(2), 129–137.

[4]  Shneiderman, B. (1996). The eyes have it: A task by data type taxonomy for information visualizations. IEEE Symposium on Visual Languages, 336–343.

[5]  ONNX Runtime. (2024). ONNX Runtime Documentation. https://onnxruntime.ai/

[6]  MongoDB Atlas. (2024). Atlas Documentation. https://www.mongodb.com/docs/atlas/

[7]  Azure OpenAI Service. (2026). GPT-5.2 Model Documentation. https://learn.microsoft.com/azure/ai-services/openai/

[8]  Espressif Systems. (2024). ESP32 Technical Reference Manual.

[9]  React. (2024). React 18 Documentation. https://react.dev/

[10]  Chart.js. (2024). Chart.js Documentation. https://www.chartjs.org/

[11]  Tailwind CSS. (2024). Tailwind CSS Documentation. https://tailwindcss.com/

[12]  scikit-learn Developers. (2024). scikit-learn 1.4 Documentation. https://scikit-learn.org/